10/100Mbps Fiber Media Converter

10/100Base-TX to 100Base-FX  ( HS130 Series)

Dual Fiber Single/ Multi Mode (Model:HS130-MSC-2/HS130-SSC-20/40/60/80/120)

Single Fiber Single/Multi Mode (Model:HS140-SSC-20/40/60/80/120)

Model:HS130-SFP

Overview

The HanXin 10/100Mbps Fiber media converters are designed to transmit and receive 10/100Mbps data over one optical fiber. The electrical interface will Auto-Negotiate to a 10Mbps, or 100Mbps Ethernet rate without any adjustments. The optical interface operates at a 100Mbps Ethernet rate. LED indicators are provided for rapidly ascertaining equipment operating status.

Key Features

1. Plug and play, high performance, Low power consumption

2. Complies with IEEE802.3 (10BASE-T) and IEEE802.3u (100BASE-TX/FX) standards

3. Auto-negotiation support on RJ45 port

4. Auto-Negotiation of duplex(HDX/FDX) on RJ45 port

5.Electrical port supports Auto-Negotiation for 10Mbps or 100Mbps, full duplex or half duplex data.

6. Auto MDI/MDI-X support on RJ45 port

7. Distances up to: 2 km Multimode, 120 km Single Mode

8. ST, SC, FC optical connectors

9. Status LED's for TX,FX LINK/ACT, POWER,100M to easily monitor network status

10.External power adapter

Applications

►Access Control Systems

►Building and Environmental Control Systems

►Tamper Switch

►High Speed Computer Links

Technical Specifications

| Parameters | Specifications |
| --- | --- |
| Standards | IEEE802.3u (100Base-TX/FX), IEEE 802.3 (10Base-T) |
| Wavelength | Multi-mode: 850 nm or 1310nm Single-mode: 1310 nm , 1550 nm |
| Transmission Distance | Fiber: Multi-mode: 2 km (fiber size: 62.5/125 or 50/125 μm) Single-mode: 20/40/60/100/120 km UTP: Cat. 3/4/5/6: 100 m |
| Port | One RJ45 port One Fiber port (ST/SC/FC Optional, LC:SFP Module) |
| MDI/MDI-X | Auto selection |
| MAC Address Table | I K |
| Buffer Size | 1Mbit |
| Flow Control | Duplex:flow control ; Half duplex:back pressure |
| Time Delay | Store-and-forward:9.6us,Straight-through:0.9us |
| BER | <1/1000000000 |
| Power Supply | Outside: 5 V DC 1 A Built-in: 100 V to 265V AC 50 Hz to 60 Hz |
| Operating Temp. | 0 °C to +70 °C |
| Storage Temp. | -20 °C to +80 °C |
| Relative Humidity | 5% to 90% (non-condensing) |
| Dimensions | 95 mm × 70 mm × 25 mm |
| Net Weight | 0.165 kg |

Connection Diagram

Ordering Information

| Model | Description | Fiber Mode | Wavelength | Distance | Connector |
| --- | --- | --- | --- | --- | --- |
| HS130-MSC-2 | 10/100Base-Tx to 100Base-FX | MMF | TRx:850nm or TRx:1310nm | 2km | SC |
| HS130-SSC-20 | 10/100Base-Tx to 100Base-FX | SMF | TRx:1310nm | 25 km | SC |
| HS130-SSC-40 | 10/100Base-Tx to 100Base-FX | SMF | TRx:1310nm | 40 km | SC |
| HS130-SSC-60 | 10/100Base-Tx to 100Base-FX | SMF | TRx:1550nm | 60 km | SC |
| HS130-SSC-80 | 10/100Base-Tx to 100Base-FX | SMF | TRx:1550nm | 80 km | SC |
| HS130-SSC-120 | 10/100Base-Tx to 100Base-FX | SMF | TRx:1550nm | 120 km | SC |
| HS130-SFP | 10/100Base-Tx to 100Base-FX |  |  |  | SFP |
| HS140-SSC-25 | 10/100Base-Tx to 100Base-FX | SMF | A side: Tx1310 nm/ Rx1550 nm B side: Tx1550 nm/ Rx1310 nm | 25 km | SC |
| HS140-SSC-40 | 10/100Base-Tx to 100Base-FX | SMF | A side: Tx1310 nm/ Rx1550 nm B side: Tx1550 nm/ Rx1310 nm | 40 km | SC |
| HS140-SSC-60 | 10/100Base-Tx to 100Base-FX | SMF | A side: Tx1310 nm/ Rx1550 nm B side: Tx1550 nm/ Rx1310 nm | 60 km | SC |
| HS140-SSC-80 | 10/100Base-Tx to 100Base-FX | SMF | A side: Tx1310 nm/ Rx1550 nm B side: Tx1550 nm/ Rx1310 nm | 80 km | SC |
| HS140-SSC-120 | 10/100Base-Tx to 100Base-FX | SMF | A side: Tx1310 nm/ Rx1550 nm B side: Tx1550 nm/ Rx1310 nm | 120 km | SC |